# USeries Data Interface Specification for

# Comms module reset

# Product:	U-Series

File Name:	U-Series_interface_document_for_comms_module_reset.doc

Author:	Vikas Jain

Classification:	Draft

Edition:	11 Aug 2021

Document History

| Date | Responsible | Description |
| --- | --- | --- |
| 11/08/2021 | Vikas Jain | Commenced Draft. |
| 24/08/2021 | Vikas Jain | Changed the event recording when |
| 31/08/2021 | Vikas Jain | Change the parameter for fixed reset and added details for parameters in events |
| 09/09/2021 | MK | Corrected the default values of power restore delay for fixed reset and SMR_Max_delay before power up |

## Table 57 (2105) – Back End Communications Configuration

| Record | Description | Offset (dec) | Length (dec) | Attribute |
| --- | --- | --- | --- | --- |
| ParameterLength | The number of bytes in the parameters array applicable to the configuration. | 0 | 2 | NVRW |
| CommsType | Selects the type of communications device. Dictates how parameter data is interpreted. | 2 | 1 | NVRW |
| Version | Specifies the version of the communications device parameters. | 3 | 1 | NVRW |
| Parameters | Configuration data, each byte is interpreted differently based on CommsType. | 4 | 256 | NVRW |
| _modem_Cellular_Fixed_Reset_t | Configuration data for the fixed reset | 260 | 8 | NVRW |
| _modem_Cellular_Smart_Reset_t | Configuration data for the smart reset | 268 | 6 | NVRW |
| Modem_Reset_Type | Configuration to decide on type of reset | 274 | 1 | NVRW |
| Spare | spare | 275 | 10 | NVRW |

### ParameterLength

#### Description

The number of bytes in the parameters array applicable to the configuration. There are up to 256 bytes of configuration data in the parameters field however the number defines how many bytes are actually used for the selected communications device.

#### Format

2 byte unsigned integer.

### CommsType

#### Description

Selects the type of communications device. Dictates how parameter data is interpreted.

#### Format

1 byte unsigned integer

| Value | Description |
| --- | --- |
| 0 | Select Utilinet for the Back End Communications system. |
| 1 | Select 3G for the Back End Communications system. |
| 2 | Select Silver Spring Networks (SSN) for the Back End Communications system. |
| 3 | Select GridNet WiMAX for the Back End Communications system. |
| 4 | Select Transparent Modem for the Back End Communications system. |
| 5 | Select RF SBS for Back End Communications system. |
| 0xFF | Unknown |

### Version

#### Description

Specifies the version of the communications device parameters. Each communication device type has its own versioning regime.

#### Format

1 byte unsigned integer.

### Parameters

#### Description

Configuration data, each byte is interpreted differently based on CommsType.

#### Format

Byte array of size 256.

### _modem_Cellular_Fixed_Reset_t

#### Description

Configuration details for the fixed reset of the cellular modem.

#### Format

| Record | Description | Offset (dec) | Length (dec) | Attribute |
| --- | --- | --- | --- | --- |
| reset_day | Reset day | 0 | 1 | NVRW |
| reset_start_time | Start time for the modem reset | 1 | 2 | NVRW |
| min_delay_reset_trigger | Minimum delay to be applied for randomization before reset action | 3 | 2 | NVRW |
| max_delay_reset_trigger | Maximum delay to be applied for randomization before reset action | 5 | 2 | NVRW |
| power_restore_delay | Delay before restoring power to the comms module | 7 | 1 | NVRW |

##### reset_day

###### Description

How often (in days) to start the power reset process. This will allow the user to preform resets on a daily, weekly, fortnightly, monthly, or quarterly cycle.

###### Format

1 byte unsigned integer.

| Configuration | Default Value(days) |
| --- | --- |
| Firmware | Min:1 Max:99 |
| Empwin+ | Min:1 Max:99 |

##### reset_start_time

###### Description

Configured time by the user to start the reset process. Default value is every midnight(00:00).

###### Format

2 byte unsigned integer.

##### min_delay_reset_trigger

###### Description

This delay is used for calculation of random delay before triggering reset of comms module.

###### Format

2 byte unsigned integer.

| Configuration | Default Value(mins) |
| --- | --- |
| Firmware | 0 |
| Empwin+ | 0 |

##### max_delay_reset_trigger

###### Description

This delay is used for calculation of random delay before triggering reset of comms module. Maximum value configurable is 24 hrs.

###### Format

2 byte unsigned integer.

Default values are mentioned below

| Configuration | Default Value(mins) |
| --- | --- |
| Firmware | 1440 |
| Empwin+ | 1440 |

##### power_restore_delay

###### Description

Delay before power restortation to the module . Default value will be 5 mins.

###### Format

1 byte unsigned integer.

Default values are mentioned below

| Configuration | Default Value(mins) |
| --- | --- |
| Firmware | Min:1 Max:30 |
| Empwin+ | Min:5 Max:30 |

### _modem_Cellular_Smart_Reset_t

#### Description

#### Format

| Record | Description | Offset (dec) | Length (dec) | Attribute |
| --- | --- | --- | --- | --- |
| smr_monitor_hour | Periodic timer for monitoring of comms activity | 0 | 2 | NVRW |
| smr_mindelay_before_pwroff | Minimum delay to be applied for randomisation | 2 | 2 | NVRW |
| smr_maxdelay_ before_pwrofff | Maximum delay to be applied for randomization | 4 | 2 | NVRW |

##### smr_monitor_hour

###### Description

Periodic timer for monitoring of comms activity. Minimum is 12 hrs and maximum is 7 days

###### Format

2 byte unsigned integer.

| Configuration | Default Value(hrs) |
| --- | --- |
| Firmware | Min:1, Max:168 |
| Empwin+ | Min:12, Max:168 |

##### smr_mindelay_before_pwroff

###### Description

This delay is used for calculation of random delay before applying power to comms module. Minimum value configurable is 5 mins.

###### Format

2 byte unsigned integer.

| Configuration | Default Value(mins) |
| --- | --- |
| Firmware | 1 min |
| Empwin+ | 5 mins |

##### smr_maxdelay_ before_pwrofff

###### Description

This delay is used for calculation of random delay before applying power to comms module. Maximum value configurable is 24 hrs.

###### Format

2 byte unsigned integer.

| Configuration | Default Value(mins) |
| --- | --- |
| Firmware | 1440 mins |
| Empwin+ | 1440 mins |

### Modem_Reset_Type

#### Description

This configuration tells the meter what type of the reset to apply to the comms module

#### Format

1 byte unsigned integer.

| Value | Description |
| --- | --- |
| 0 | None. (reset disabled) |
| 1 | Smart |
| 2 | Fixed |
| 3 | Smart + Fixed |

## Events

### Remote Power Fail (2113)

### Description

This event is used to indicate that the power fail pin on the remote port has been asserted. This typically occurs when the meter experiences a loss of mains supply.

For normal remote power fail there is no argument recorded. If this event is due to coms reset process then 1 byte argument will be recorded -

#### Description

Byte 0: which mode has triggered the reset process.

| Value | Description |
| --- | --- |
| 1 | Smart |
| 2 | Fixed |