HCM Testing

To Search IHD device already in HCM

With partial MAC Address information use wildcard, e.g., for XXXX045E MAC address go to

Devices » Search » HAN Device » enter *045E in MAC Address textbox under Common Fields » Search » Device Search Results are shown with a MAC Address link to the HAN Device Summary page

To Search Meter already in HCM

With partial MAC Address information use wildcard, e.g., for XXXX78C0 MAC address go to

Devices » Search » ESI » enter *78C0 in MAC Address textbox under Common Fields » Search

To add a new IHD into HCM

To add a new meter into HCM

To check whether a meter has HAN certificate

Search the known meter as shown above. On Device Search Results page click on the link of the device MAC address. Click on Provision tab to open Enable HAN Radio window » Activate (if HAN Radio is not yet activated). Once activation is completed close the Enable HAN Radio window.

Click on the Sync tab. Once the sync activity is completed check whether Certificate Type under ZigBee Attributes is or changes to Production. (NB: None means that the meter has no HAN certificate. “Production” means that the meter has HAN certificate. The HAN certificate has to be requested through UiQ support if it is not there).

To associate Meter to IHD

Search the IHD as described above

Enrol Program to a Meter

Programs » List » Click on any Program Name link » Customers » Enroll Customers » Choose File (HCM Customer Enrol) » Open » Close » Sync

To Enable HAN Radio

HAN to Join Network

# Sequesnce of Testing:

- Associate to IHD

- Enrol a program

- Provision