| Description | Command | Result |
| --- | --- | --- |
| Use “net_mgrS -d” instead of “-d” for UE MST tool commands |  |  |
| Image list | -d fd73:e9ec:c000:0:213:5002:9:b044 image list |  |
| Verify image signature | -d fd73:e9ec:c000:0:213:5002:9:b044 image image_signature 03.10.0001 | Status: 0; rc: 0 (BOOT_OK) |
| Swap NIC image to run 03.10.0001 | -d fd73:e9ec:c000:0:213:5002:9:b044 image setboot 03.10.0001 03.10.0001 |  |
| Restart/reboot | -d fdd7:24b:500:2:213:5005:40:3263 restart now gasp wait 1 -d fdd7:24b:500:2:213:5005:40:3263 restart gasp in_secs 3 |  |
| Set boot force | -d fd73:e9ec:c000:0:213:5002:9:b044 image setboot force 03.10.0001 03.10.0001 |  |
| Operator cert get | -d fd73:e9ec:c000:0:213:5002:9:b044 certs sdump 4 |  |
| Upload cert | -d fd73:e9ec:c000:0:213:5002:9:b044 certs upload /home/ssn/UIQ-DEV-broadcast-23-02-23.x509 persist -d fdd7:24b:500:2:213:5003:57:78a4 certs upload /home/ssn/lab-broadcast-22-11-2022.x509 persist -d [device] certs upload /home/ssn/PROD-broadcast-02-03-2025.x509 persist | /home/ssn/UIQ-DEV-broadcast-23-02-23.x509 ask for the path from UiQ Support (Nelson) |
| Sync cert | -d fd73:e9ec:c000:0:213:5002:9:b044 certs sync |  |
| Password get | -d fd73:e9ec:c000:0:213:5002:9:b044 c1219_password |  |
| Password change | -d fd73:e9ec:c000:0:213:5002:9:b044 c1219_password xxxxxxxxxxx |  |
| SIQ Commands |  |  |
| To check the SIQ status | -d fd73:e9ec:c000:0:213:5002:b:9bf2 power_mon status | Provides Profile IDs |
| Read the SIQ log from Device | -d fd89:1e09:aa94:2ac:213:5003:f:1925 -t 10 power_mon native_log profile_id:1029 read 1 |  |
| Read the range of log profile | -d fd89:1e09:aa94:2ac:213:5003:f:1925 -t 10 power_mon native_log profile_id:1029 read <start_seq> <end_seq> |  |
| SIQ Alarm Read | -d fd73:e9ec:c000:0:213:5005:d9:6611 power_mon alarm_log profile_id:1098 read |  |
| Enables time sync | -d <ip address> c1219_ts_enable 1 |  |
| Enables setting of the date | -d <ip address> c1219_ts_set_date 1 |  |
| Force time and date set | -d <ip address> c1219_ts_trigger force_date |  |
| Read SIQ profile in Epoch time range | -d fd73:e9ec:c000:0:213:5005:ff3f:917f power_mon native_log profile_id:1029 read -e 1738625100 1738627100 |  |
| Battery Test AP | -d fd89:1e09:aa00:246:213:50ff:fe71:d032 battery test |  |
| Get Net Manager Version | get_version_str -V |  |
| Help | -d fd73:e9ec:c000:0:213:5005:ff3f:917f power_mon |  |
| Device is responding on address: fdd7:24b:500:2:213:5005:40:3263 Continue executing Net Mgr command -d fdd7:24b:500:2:213:5005:40:3263 image image_list Command: -d fdd7:24b:500:2:213:5005:40:3263 image image_list Usage: net_mgr [-h cmd] [-d] v6¦-i <rf_itf> image -h cmd help for a command, or general help -d address of the NIC you are messaging -i RF interface you are messaging Possible image commands are: remove [version] (version is major.minor.revision) setboot [lkg] <cri> [force] (image versions) list - list the images list r - (same as list but output format is major.minor.revision) list_env - extra data about image build environment (if available) upload <version - image to upload> [offset] [msglen] get_chunk <image to get from> [offset] [msglen] get_image <version> [offset] [msgsize] [valid](valid option set: only get valid images) get_from_neighbors <version> [delay - in seconds] [all] (tell nodes to ask neighbors for image) stop_get_from_neighbors <version> compat_neighbors corelist - list the cores coreremove <version> core_hello coreload <version - core to download> [offset] [msgsize] device_list - show device vendor info, include hosted slots device_list legacy_fmt - device_list with old format device_remove [version] device_get_image <version> meter_list (deprecated - use device_list) meter_remove [version] (deprecated - use device_remove) meter_get_image <version> (deprecated - use device_get_image) image_verify <version> image_crc <version> image_signature <version> image_verify_signatures <version> image_list_signatures <version> image_signature_boot2 <version> image_verify_signatures_boot2 <version> image_list_signatures_boot2 <version> host_image <version - image to upload> [offset] [msglen] get_host_image <version> [offset] [msgsize] [valid](valid option set: only get valid images) host_from_neighbors <version> arm¦akio¦rni¦g5riva [delay - in seconds] [all] (ask neighbors for image to host) stop_host_from_neighbors <version> arm¦akio¦rni¦g5riva meter_from_neighbors <version> i210¦meter [delay - in seconds] [all] (ask neighbors for meter image) (deprecated - use device_from_neighbors) stop_meter_from_neighbors <version> i210¦meter (deprecated - use stop_device_from_neighbors) device_from_neighbors <version> i210¦meter [delay - in seconds] [all] (ask neighbors for meter image) stop_device_from_neighbors <version> i210¦meter hosted_device_from_neighbors <version> i210¦meter [delay - in seconds] [all] (ask neighbors for meter image in hosted image slot) stop_hosted_device_from_neighbors <version> i210¦meter WARN net_mgrS.c:2754 /usr/ssn/net_mgr/net_mgr_lite returned 256 |  |  |