Professional Certificate of Competency in Electrical Power System Fundamentals for Non-electrical Engineers (CEN)

| Student full name: | Manoj Kalra |  |  |  |
| --- | --- | --- | --- | --- |
| I certify that the attached assessment is my own work and that any material drawn from other sources has been acknowledged. Copyright in assessments remains my property, however I grant permission to the Engineering Institute of Technology (EIT) to make copies of assessments for assessment, review and/or record keeping purposes. I note that the EIT reserves the right to check my assessment for plagiarism. Should the reproduction of all or part of an assessment be required by the EIT for any purpose other than those mentioned above, appropriate authorisation will be sought from me on the relevant form. |  |  |  |  |
| Please place a tick () in the box below to indicate that you have read, understood, and certify the above statement. Please include this page in/with your submission. Any electronic responses to this submission will be sent to your Moodle account. AGREEMENT DATE: 23-Oct-2024 |  |  |  |  |
| Marks (%) |  |  | Satisfactory / Not Satisfactory |  |
| Assessor: |  |  | Date: |  |
| Overall feedback: |  | Please see feedack next to the respective answers |  |  |

Guidelines for Students

How is this assessed?

After completion of this assessment, you will be given a result of ‘Satisfactory’ or ‘Not Satisfactory’. The assessor will give you feedback via Moodle and you will have an opportunity to submit additional evidence, if you have received a ‘Not Satisfactory’ result.

You will be allowed one (1) opportunity to resubmit the same assessment task, if required.

For a ‘Satisfactory’ result in this assessment, all questions must be answered to a satisfactory standard and you must achieve  an overall mark of 60% or above.

Once all assessment tasks for have been completed, you will be given a final result of ‘Competent’ or ‘Not Yet Competent’. If you are deemed ‘Not Yet Competent’ in a course after all resubmission attempts, you will be required to re-sit the course.

How is this assessment task assessed?

For a result of ‘Satisfactory’ in this assessment task, all assessment criteria (as indicated on page 4) must be completed  to a satisfactory standard.

Where a critical question is identified, you must receive a mark of 100% for these questions before a ‘satisfactory’ result can be awarded, regardless of the overall mark achieved.

At Certificate level, a ‘satisfactory’ standard, as stipulated by the Australian Qualifications Framework, means that you will demonstrate the application of knowledge and skills:

- with depth in areas of specialisation, in contexts subject to change

- with initiative and judgment in planning, design, technical or management functions with some direction

- to adapt a range of fundamental principles and complex techniques to known and unknown situations

- across a broad range of technical or management functions with accountability for personal outputs

- personal and team outcomes within broad parameters

Assessors also make decisions based on the following considerations:

- all parts of this assessment have been completed to a standard that satisfactorily meets the requirements set out in the assessment criteria (as per the course outline).

- the assessment evidence provided is the student’s own work, except as appropriately acknowledged by the use of referencing.

- the evidence is recent and the student’s knowledge is up-to-date

Assessment Instructions:

- You must answer ALL questions.

- Please ensure you complete your answers in a blue font (not red or black).

- The best marks can be earned by giving concise, brief answers that address the questions.

- You must reference all content used from other sources including course materials, slides, diagrams, etc. Do not directly copy and paste from course materials or any other resources. 
Refer to the referencing section of the EIT eLibrary on Moodle for referencing guides.

- Use this document for completing your answers by typing the answers after each question without deleting the question. Make sure that you preserve the original question number format.

- Do not add extra pictures, etc. as annexures; instead, paste them directly into this answer sheet. Hand-drawn sketches can be inserted after scanning but please ensure that the file size does not become big (more than 10 MB). You must refer to all diagrams and pictures, etc. that you have drawn or pasted in.

- When saving your document (must be Word format), ensure you include your name in the title: COURSECODE_ASSESSMENTTYPE_VERSION#_YOURNAME

E.g. CEN_M5-9_Assessment2_v1_JohnSmith

| Modules: | 6-9 |
| --- | --- |
| Assessment type: | Assessment 2 |
| Version: | 1 |
| Total marks: | 50 marks |

Learning outcomes covered in this assessment:

Module 6: Faults, fuses and circuit breakers

Module 7: Power Systems Protection and Electrical Safety

Module 8: Load Forecasting, Planning and Project Evaluation

Module 9: Impacts of renewables on grid and Power Quality

PART 1: Short-answer questions 	              				 (70 marks)

| Q1 | List the types of active faults. Also list one examples for each type | (2 marks) |
| --- | --- | --- |
| A1 | Student answer: There are 2 types of active faults: - Soli Faults. Example: Short Circuit - Incipient Faults. Example: Partial Discharge |  |
| F1 | Assessor feedback |  |

| Q2 | What do you understand by energy let through? How do fuses perform with regards to energy let through compared to circuit breakers? | (2 marks) |
| --- | --- | --- |
| A2 | Student answer: Energy let through is referred as the energy let into the circuit till fusing. The amount of energy is proportional to I^2*t. Fuses are able to limit the let through energy by fusing very quickly and the time is usually under 1/4th of ac cycle; equivalent to ¼(20) = 5 mSec. Whereas circuit breaker usually take longer time of about 10 ac cycles; equivalent to 10*20 = 200mSec. |  |
| F2 | Assessor feedback |  |

| Q3 | Define: - Isolation - Switching | (2 marks) |
| --- | --- | --- |
| A3 | Student answer: - Isolation means cutting off the supply from all or discrete section of the installation by separating the installation from every source of energy for the safety purposes. - Switching means closing or opening of an electric circuit or to divert a load from one circuit to another which carries high current. |  |
| F28 | Assessor feedback |  |

| Q4 | List is the term used for automatic operation of a circuit breaker through its operating mechanism in case of a short circuit or overload condition. | (2 marks) |
| --- | --- | --- |
| A4 | Student answer: Tripping |  |
| F4 | Assessor feedback |  |

| Q5 | List one advantage and one disadvantage of a vacuum circuit breaker. | (2 marks) |
| --- | --- | --- |
| A5 | Student answer: Advantage: Vacuum circuit breakers are relatively inexpensive and compact. Disadvantage: They are generally limited to 36 KV. |  |
| F5 | Assessor feedback |  |

| Q6 | List the four basic qualities of modern protection relays (The 4Ss of protection). |  | (2 marks) |
| --- | --- | --- | --- |
| A6 | Student answer: - Selectivity - Stability - Sensitivity - Speed |  |  |
| F6 | Assessor feedback |  |  |
| Q7 |  | List the two basic protection types | (2 marks) |
| A7 |  | Student answer: - Definite Minimum Time (DMT) - Inverse Definite Minimum Time (IDMT) |  |
| F7 |  | Assessor feedback |  |
| Q8 |  | List any two means of preventing arc flash hazards by elimination technique | (2 marks) |
| A8 |  | Student answer: - Insulate the exposed metals part - By blocking entry of rodents and reptiles |  |
| F8 |  | Assessor feedback |  |
| Q9 |  | List any two ways of reducing the level of arc energy released. | (2 marks) |
| A9 |  | Student answer: - By reducing the fault current - Using the fast acting protection mechanism which ill reduce the exposure time |  |
| F9 |  | Assessor feedback |  |
| Q10 |  | List the mandatory tiers enforceable by law in the three tiers of legislation | (2 marks) |
| A10 |  | Student answer: - Act - Regulation - Code of Practice |  |
| F10 |  | Assessor feedback |  |
| Q11 |  | List any four needs for power system planning | (2 marks) |
| A11 |  | Student answer: - Electrical distribution system optimisation - Make optimum utilization of resources while meeting the requirements. - Safe working environment for resources and trouble-free services. - Freedom from unplanned interruptions. |  |
| F11 |  | Assessor feedback |  |

| Q12 | Least any four typical studies needed in planning | (2 marks) |
| --- | --- | --- |
| A12 | Student answer: - Load Estimation - Load Flow - Fault Level - Voltage Profile |  |
| F12 | Assessor feedback |  |

| Q13 | - Define smart grid. - List any two of its benefits. | (2 marks) |
| --- | --- | --- |
| A13 | Student answer: - Smart Grid is a combination of hardware, management and reporting software, with intelligent communications infrastructure. - Benefits: - Minimum power disturbance if any, quick restoration - Two way flow of electricity and data |  |
| F13 | Assessor feedback |  |

| Q14 | List any four key aspects of substation automation. | (2 marks) |
| --- | --- | --- |
| A14 | Student answer: - Protection - Control - Measurement - Monitoring |  |
| F14 | Assessor feedback |  |

| Q15 | - What do you understand by the term ‘carbon credit’? - What is the intention of such a credit? | (2 marks) |
| --- | --- | --- |
| A15 | Student answer: - Carbon credit in a generic term is a tradeable certificate or permit given for a company, permitting to emit one ton of carbon dioxide or the mass of another greenhouse gas equivalent to one ton of carbon dioxide. - The intention of carbon credit is to drive industrial and commercial processes towards low emissions or less carbon intensive avenues. |  |
| F15 | Assessor feedback |  |

| Q16 | - What is the difference between sag and swell? - How long do short duration sag and swell usually persist in AC systems? | (2 marks) |
| --- | --- | --- |
| A16 | Student answer: - Sag is a decrease of the rms voltage or current between 0.1 and 0.9 p.u. at the power frequency, whereas, swell is an increase of the rms voltage or current between 1.1 and 1.8 p.u. at the power frequency. - duration from 0.5 cycles(10 msec) to 1 min |  |
| F16 | Assessor feedback |  |
| Q17 | List any two issues if frequency fluctuates outside tolerances. | (2 marks) |
| A17 | Student answer: - It causes poor speed regulation of local generation. - It generates faults on the bulk power systems |  |
| F17 | Assessor feedback |  |
| Q18 | List any two origins of lightning surges. | (2 marks) |
| A18 | Student answer: - Due to induced over voltages on overhead lines - Due to direct flashes to overhead lines |  |
| F18 | Assessor feedback |  |
| Q19 | List any two ways of managing voltage variations | (2 marks) |
| A19 | Student answer: - By using the voltage regulating devices i.e. UPS - By addition of generating capacity i.e. tie feeders close to load bus |  |
| F19 | Assessor feedback |  |

Part 2: Long-answer question 		  (12 marks)

| Q20 | In your own words please summarise the challenges that electricity networks face when integrating new customer generation connections into the grid. How are those challenges addressed in terms of mitigating the associated impacts? | (6+6=12 marks) |
| --- | --- | --- |
| A20 | Student answer: - When integrating new customers generation connections to the grid, there are following challenges faced: - Intermittent nature of renewable energy, which can cause the fluctuations. - Difficulty in matching the demand with supply due to more renewable power being flown on the distribution network. - Need additional supply security measures e.g. energy storage. - Since weather can change quickly and e.g. it may happen so that the customer is generating the energy and supplying to the network, but all of sudden due to weather change, customer does not generate the energy and instead seeks for the energy from the distribution network, which if happens in bulk may disbalance the load on the network and utility distributor need to maintain the net load quickly. - Such scenarios as discussed above, may cause volage variations which needs extra cost and complexity in the network. - Mitigation the associated impacts: - By providing the control mechanism to the distributor to turn off the customer generation being supplied to the network i.e. using smart inverters - Installing voltage regulating devices - Transformer on load tap adjustment - By designing the system to regulate the grid voltage with in the allowable limits by consuming the surplus energy or by regulating the reactive power - Energy storage: electricity can be stored during the times of high generation for later usage. - Building mor transmission line: the energy being generated from high renewable resources area can be supplied to high energy demand areas - Demand side management: shifting flexible demand to times of high renewable production, i.e. preheating electric hot water heaters |  |
| F20 | Assessor feedback |  |

END OF ASSESSMENT